# Supplementary Information-II

Python script for computing the parameters

import numpy as np

#Creating an

Ti = np.array([i for i in range(8)])   #Creating an array for titanium with 1 to 7 numbers.

Ti = Ti.astype(np.float64)

#Now replace those 1 to 7 numbers with the melting point, electronegativity, atomic radius, density, and molar volume.

Ti[0] = 1668  #Melting point of titanium

Ti[1] = 1.54  #Electro negativity of titanium

Ti[2] = 0.14  #Atomic radius of titanium

Ti[3] = 4.507 #Density of titanium

Ti[4] = 10.4  #Molar volume of titanium

# the Same process as above is repeated for all 11 elements.

V = np.array([i for i in range(8)])

V = V.astype(np.float64)

V[0] = 1910

V[1] = 1.63

V[2] = 0.135

V[3] = 6.11

V[4] = 8.32

Cr = np.array([i for i in range(8)])

Cr = Cr.astype(np.float64)

Cr[0] = 1907

Cr[1] = 1.66

Cr[2] = 0.14

Cr[3] = 7.14

Cr[4] = 7.23

Zr = np.array([i for i in range(8)])

Zr = Zr.astype(np.float64)

Zr[0] = 1855

Zr[1] = 1.33

Zr[2] = 0.155

Zr[3] = 6.511

Zr[4] = 14.02

Nb = np.array([i for i in range(8)])

Nb = Nb.astype(np.float64)

Nb[0] = 2477

Nb[1] = 1.6

Nb[2] = 0.145

Nb[3] = 8.57

Nb[4] = 10.83

Mo = np.array([i for i in range(8)])

Mo = Mo.astype(np.float64)

Mo[0] = 2623

Mo[1] = 2.16

Mo[2] = 0.145

Mo[3] = 10.28

Mo[4] = 9.38

Hf = np.array([i for i in range(8)])

Hf = Hf.astype(np.float64)

Hf[0] = 2233

Hf[1] = 1.3

Hf[2] = 0.155

Hf[3] = 13.31

Hf[4] = 13.44

Ta = np.array([i for i in range(8)])

Ta = Ta.astype(np.float64)

Ta[0] = 3017

Ta[1] = 1.5

Ta[2] = 0.145

Ta[3] = 16.65

Ta[4] = 10.85

W = np.array([i for i in range(8)])

W = W.astype(np.float64)

W[0] = 3422

W[1] = 2.36

W[2] = 0.135

W[3] = 19.25

W[4] = 9.47

Al = np.array([i for i in range(8)])

Al = Al.astype(np.float64)

Al[0] = 660.32

Al[1] = 1.61

Al[2] = 0.125

Al[3] = 2.7

Al[4] = 10

Si = np.array([i for i in range(8)])

Si = Si.astype(np.float64)

Si[0] = 1414

Si[1] = 1.90

Si[2] = 0.11

Si[3] = 2.33

Si[4] = 12.06

f = open ("Alloys.dat", "w")

f.write("Alloy Melting_Point Electro_Negativity Atomic_mismatch\n\n")

for i in range(300):

Al[6] = input("\n Al: ")

Cr[6] = input("\n Cr: ")

Si[6] = input("\n Si: ")

Ti[6] = input("\n Ti: ")

V[6] = input("\n V: ")            #Asks an input (atomic % of each element) from the user for every element.

Zr[6] = input("\n Zr: ")

Nb[6] = input("\n Nb: ")

Mo[6] = input("\n Mo: ")

Hf[6] = input("\n Hf: ")

Ta[6] = input("\n Ta: ")

W[6] = input("\n W: ")

x = 1/100

Ti[7] = Ti[6]*x

V[7] = V[6]*x

Cr[7] = Cr[6]*x

Zr[7] = Zr[6]*x

Nb[7] = Nb[6]*x

Mo[7] = Mo[6]*x

Hf[7] = Hf[6]*x

Ta[7] = Ta[6]*x

W[7] = W[6]*x

Al[7] = Al[6]*x

Si[7] = Si[6]*x

Melting_Point = ((Si[7])*Si[0]) + ((Ti[7])*Ti[0]) + ((V[7])*V[0]) + ((Cr[7])*Cr[0]) + ((Zr[7])*Zr[0]) + ((Nb[7])*Nb[0]) + ((Mo[7])*Mo[0]) + ((Hf[7])*Hf[0]) + ((Ta[7])*Ta[0]) + ((W[7])*W[0]) + ((Al[7])*Al[0])

Electro_Negativity = ((Si[7])*Si[1]) + ((Ti[7])*Ti[1]) + ((V[7])*V[1]) + ((Cr[7])*Cr[1]) + ((Zr[7])*Zr[1]) + ((Nb[7])*Nb[1]) + ((Mo[7])*Mo[1]) + ((Hf[7])*Hf[1]) + ((Ta[7])*Ta[1]) + ((W[7])*W[1]) + ((Al[7])*Al[1])

r = ((Si[7])*Si[2]) + ((Ti[7])*Ti[2]) + ((V[7])*V[2]) + ((Cr[7])*Cr[2]) + ((Zr[7])*Zr[2]) + ((Nb[7])*Nb[2]) + ((Mo[7])*Mo[2]) + ((Hf[7])*Hf[2]) + ((Ta[7])*Ta[2]) + ((W[7])*W[2]) + ((Al[7])*Al[2])

Atomic_Mismatch = (Si[7]*((1-(Si[2]/r))**2) + Ti[7]*((1-(Ti[2]/r))**2) + V[7]*((1-(V[2]/r))**2) + Cr[7]*((1-(Cr[2]/r))**2) + Zr[7]*((1-(Zr[2]/r))**2) + Nb[7]*((1-(Nb[2]/r))**2) + Mo[7]*((1-(Mo[2]/r))**2) + Hf[7]*((1-(Hf[2]/r))**2) + Ta[7]*((1-(Ta[2]/r))**2) + W[7]*((1-(W[2]/r))**2) + Al[7]*((1-(Al[2]/r))**2))**(0.5)

Density = ((Si[7])*Si[3]) + ((Ti[7])*Ti[3]) + ((V[7])*V[3]) + ((Cr[7])*Cr[3]) + ((Zr[7])*Zr[3]) + ((Nb[7])*Nb[3]) + ((Mo[7])*Mo[3]) + ((Hf[7])*Hf[3]) + ((Ta[7])*Ta[3]) + ((W[7])*W[3]) + ((Al[7])*Al[3])

Molar_Volume = ((Si[7])*Si[4]) + ((Ti[7])*Ti[4]) + ((V[7])*V[4]) + ((Cr[7])*Cr[4]) + ((Zr[7])*Zr[4]) + ((Nb[7])*Nb[4]) + ((Mo[7])*Mo[4]) + ((Hf[7])*Hf[4]) + ((Ta[7])*Ta[4]) + ((W[7])*W[4]) + ((Al[7])*Al[4])

f.write("%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f %f %f %f %f %f %f %.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f%.2f\n" %(Ti[6],V[6],Cr[6],Zr[6],Nb[6],Mo[6],Hf[6],Ta[6],W[6],Al[6],Si[6],Melting_Point,Electro_Negativity,Atomic_Mismatch,Density,Molar_Volume,Ti[7],V[7],Cr[7],Zr[7],Nb[7],Mo[7],Hf[7],Ta[7],W[7],Al[7],Si[7]))

print("\nMelting_Point: {:.3f}".format(Melting_Point)+"\n\nElectro_Negativity: {:.3f}".format(Electro_Negativity)+"\n\nAtomic_Mismatch: {:.3f}".format(Atomic_Mismatch) + "\n\nDensity: {:.3f}".format(Density) + "\n\nMolar_Volume: {:.3f}".format(Molar_Volume) + "\n\nAl: {:.2f}".format(Al[7]*100)+"\nCr: {:.2f}".format(Cr[7]*100)+"\nSi: {:.2f}".format(Si[7]*100)+"\nTi: {:.2f}".format(Ti[7]*100)+"\nV: {:.2f}".format(V[7]*100)+"\nZr: {:.2f}".format(Zr[7]*100)+"\nNb: {:.2f}".format(Nb[7]*100)+"\nMo: {:.2f}".format(Mo[7]*100)+"\nHf: {:.2f}".format(Hf[7]*100)+"\nTa: {:.2f}".format(Ta[7]*100)+"\nW: {:.2f}".format(W[7]*100))

f.close()